電子學(二) Integrated-Circuit Amplifiers

自主學習    作業2

- Use the Bode plot to draw the magnitude response and the phase response of each of the following transfer functions.

- Ts=10s+1000s+100

- Ts=20s2s+100

- Ts=105s+103s+2×104

- Consider a voltage amplifier having a frequency response of the low-pass STC(single-time-constant) type with a dc gain of 40 dB and a 3-dB frequency of 2000 Hz. Find the gain in dB at f = 20 Hz, 2 kHz, 20 kHz, and 2 MHz. Also, express the transfer function for the amplifier.

- Consider the circuit in Fig. 1. Let Rs = 20 k and Ri = 80 k. Convince yourself that V2/Vs is a high-pass STC function. What is the smallest value for C that will ensure that the 3-dB frequency is not higher than 100 Hz?

Fig. 1